DNA extraction from animal blood protocol

Standard Operating Procedure #I.02 v.20260221

AG Sociality & Evolution (Kopp), University of Konstanz

Responsible: 		Dr. Gisela H. Kopp

Date created: 		2025-07-23

Created by: 		Gisela Kopp

Revision #:		1

Revision Date:	 	21.02.2026

Revised by:		Gisela Kopp

Major points of rev.:	RNase treatment, incubation times

Related protocols:

- NA

# SOP #I.02: DNA extraction from blood protocol

Protocol for extracting DNA from blood with Qiagen DNeasy Blood & Tissue Kit.

Based on DNeasy Blood & Tissue Handbook’s protocol for Purification of Total DNA from Animal Blood (spin-column protocol)

## Preconditions and lab environment

- You need to have received lab instructions before starting to work in the lab

  - Z838 (Dr. Gisela H.Kopp, gisela.kopp@uni-konstanz.de)

  - Limnological Institute (Dr. Azade Voolstra, azade.voolstra@uni-konstanz.de)

  - MRC (N.N.)

- Follow Betriebsanweisung_Z838_2022 and Risk_Assessment_biologicalsamples for working in clean room and with biological samples to ensure decontaminated work space.

- Wear lab coat with cuffed sleeves, sleeve covers, safety glasses, disposable mask and extended gloves throughout the whole procedure. It is critical to avoid contamination.

- Keep a detailed protocol in the hardcopy lab book (needs to stay in the lab) and in the online lab book “labfolder”.

- Note all dates and results in the project overview table.

- Give notice if lab materials run low.

### Time estimate:

- 3 h DNA extraction for 29 samples

Tip: use uneven number of samples to have balanced centrifuge when adding the negative control.

## Required instruments and materials

|  | Manufacturer & Article number | Amount total [t]/ per sample [ps] | location |
| --- | --- | --- | --- |
| Equipment |  | total |  |
| Pipettes (volumes 10ul, 100ul, 200ul, 300ul, 1000ul) |  | 1 |  |
| Clean hood |  | 1 |  |
| Microcentrifuge with rotor for 1.5ml and 2ml tubes |  | 1 |  |
| Vortex |  | 1 |  |
| Heating block or Thermomixer |  | 1 |  |
| Epi rack |  | 2 |  |
| Plastic ware |  | Per sample |  |
| Pipette filter tips 10ul |  | 1 |  |
| Pipette filter tips 100ul | Sarstedt | 2 |  |
| Pipette filter tips 200ul | Sarstedt | 4 |  |
| Pipette filter tips 300ul | Sarstedt | 2 |  |
| Pipette filter tips 1000ul | Sarstedt | 2 |  |
| 1.5ml tubes | Sarstedt | 1 (+3) |  |
| 2ml tubes | Sarstedt | 1 |  |
| Collection tubes | Qiagen (1000) #19201 | 1 |  |
| Reagents |  | Per sample |  |
| DNeasy Blood and Tissue Kit [incl. DNeasy Mini Spin Columns, 2ml collection tubes, ATL buffer, AL buffer, AW1 buffer concentrate, AW2 buffer concentrate, AE buffer, Proteinase K] | Qiagen (50) #69504 (250) #69506 | 1 kit for 50 or 250 samples | RT, Proteinase K at 2-8°C for storage >1year |
| Ethanol 100% |  | 200ul | RT |
| PBS pH 7.2 (50mM potassium phosphate, 150mM NaCl) |  | 100ul | 2-8°C |
| RNase A 100mg/ml | Qiagen 2.5ml #19101 | 20ul |  |

## Procedure

Extraction number: _________________ Date:_____________ 	Elution buffer:________________________	Elution volume: __________________

Personnel: ______________________________________		Sample Information: Number of samples (=n): ________

| Sample ID | Aliquot No | Species | Collection Date | Collection location | DNA extraction date | Sample volume | DNA extract ID | DNA conc. (ng/ul) | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Comments:

### Before starting

- Make sure that the appropriate amounts (as indicated on bottle) of ethanol were added to the AW1 and AW2 buffer concentrates.

- If Buffer AL has formed a precipitate, warm to 56°C to dissolve

- Preheat the heating block or thermomixer to 56°C

### Lysis

- Pipet into labelled 1.5ml tube: 20ul Proteinase K

- Add 100ul anticoagulated blood

*Note: Blood from mammals contains non-nucleated erythrocytes. Blood from other animals, e.g. birds, contains nucleated erythrocytes, in these cases reduce volume to 10ul and increase PBS in next step.

- Add 100ul PBS to reach final volume of 220 ul

- To obtain RNA-free genomic DNA, add 4ul RNase A (100mg/ml) and incubate for 2min at RT

- Add 200ul Buffer AL

Mix thoroughly by vortexing

*Note: ensure that no ethanol has been added to Buffer AL. It is essential that the sample and Buffer AL are mixed immediately and thoroughly to yield a homogenous solution.

- Incubate 20min at 56°C shaking. (If no shaking is available, vortex every 5min).

Sample should be completely lysed, and duration might depend on the sample.

Use incubation time to place DNeasy Mini Spin Columns in 2ml collection tube and label

- spin down

Add 200ul 99%EtOH

Mix thoroughly by vortexing to yield a homogeneous solution

### Binding and Washing

- Pipet the ca. 650ul mixture into spin column

Centrifuge 8000rpm for 1min

Discard collection tube with flow-through

*Note: Flow-through contains Buffer AL or Buffer AW1 and is therefore not compatible with bleach.

- Place spin column into new collection tube

Add 500ul Buffer AW1

Centrifuge 8000rpm for 1min

Discard collection tube with flow-through

- Place spin column into new collection tube

Add 500ul Buffer AW2

Centrifuge 14,000rpm for 3min

Discard collection tube with flow-through

- Place spin column into new collection tube

Centrifuge 14,000rpm for 1min to dry membrane

Discard collection tube with flow-through

*Note: it is important to dry the membrane of the spin column, since residual ethanol may interfere with subsequent reactions.

### Elution

- Place spin column into clean, labelled 1.5ml tube

- Pipet 110ul Buffer AE onto membrane

Incubate 10-20min at RT

Centrifuge 8000rpm for 1min

- Again pipet 110ul Buffer AE onto membrane

Incubate 10-20min at RT

Centrifuge 8000rpm for 1min

- Prepare 4 aliquots of 50ul each to prevent freeze-thaw cycles in downstream use.

- Label all tubes (incl “DNA”, DNA extract ID; date, storage buffer) and included in databases.

- Store DNA eluate at 2-8°C for direct further use, at -20°C for mid-term storage, at -80°C for long-term storage.

## Continue with:

### DNA quantification and quality

- Qubit dsDNA BR assay

    - SOP #IV.01

  - TapeStation

    - SOP XXX

*Note: Expected DNA amount ca. 5ug = 5000ng, i.e. 22.7 ng/ul for 220ul eluate.

### DNA library preparation and sequencing

- Send DNA to sequencing facility

    - SOP XXXX