DNA extraction from hair – option B - protocol

Standard Operating Procedure #I.04 v.20250723

AG Sociality & Evolution (Kopp), University of Konstanz

Responsible: 		Dr. Gisela H. Kopp

Date created: 		2025-07-23

Created by: 		Gisela Kopp

Revision #:		0

Revision Date:	 	NA

Revised by:		NA

Major points of rev.:	NA

Related protocols:

- NA

# SOP #I.04: DNA extraction from hair – option B - protocol

Protocol for extracting DNA from hair with QIAamp DNA Investigator Kit.

Based on QIAamp DNA Investigator Handbook’s protocol for Isolation of Total DNA from Nail Clippings and Hair.

## Preconditions and lab environment

- You need to have received lab instructions before starting to work in the lab

  - Z838 (Dr. Gisela H.Kopp, gisela.kopp@uni-konstanz.de)

  - Limnological Institute (Dr. Azade Voolstra, azade.voolstra@uni-konstanz.de)

  - MRC (N.N.)

- Follow Betriebsanweisung_Z838_2022 and Risk_Assessment_biologicalsamples for working in clean room and with biological samples to ensure decontaminated work space.

- Wear lab coat with cuffed sleeves, sleeve covers, safety glasses, disposable mask and extended gloves throughout the whole procedure. It is critical to avoid contamination.

- Keep a detailed protocol in the hardcopy lab book (needs to stay in the lab) and in the online lab book “labfolder”.

- Note all dates and results in the project overview table.

- Give notice if lab materials run low.

### Time estimate:

- 2 h DNA extraction

## Required instruments and materials

|  | Manufacturer & Article number | Amount total [t]/ per sample [ps] | location |
| --- | --- | --- | --- |
| Equipment |  | total |  |
| Pipettes (volumes 100ul, 200ul, 300ul, 1000ul) |  | 1 |  |
| Clean hood |  | 1 |  |
| Microcentrifuge with rotor for 1.5ml and 2ml tubes |  | 1 |  |
| Vortex |  | 1 |  |
| Heating block or Thermomixer |  | 1 |  |
| Epi rack |  | 2 |  |
| Scissor or appropriate cutting device |  |  |  |
| Plastic ware |  | Per sample |  |
| Pipette filter tips 100ul | Sarstedt | 2 |  |
| Pipette filter tips 200ul | Sarstedt | 4 |  |
| Pipette filter tips 300ul | Sarstedt | 2 |  |
| Pipette filter tips 1000ul | Sarstedt | 2 |  |
| 1.5ml tubes | Sarstedt | 1 (+3) |  |
| 2ml tubes | Sarstedt | 1 |  |
| Collection tubes | Qiagen (1000) #19201 | 1 |  |
| Reagents |  | Per sample |  |
| QIAamp DNA Investigator Kit [incl. QIAamp MinElute Columns, 2ml collection tubes, ATL buffer, AL buffer, AW1 buffer concentrate, AW2 buffer concentrate, ATE buffer, Carrier RNA, Proteinase K] | Qiagen (50) #56504 | 1 kit for 50 samples | RT, Columns and Proteinase K at 2-8°C for storage >1year |
| Ethanol 100% |  | 200ul | RT |
| Optional: RNase A 100mg/ml | Qiagen 2.5ml #19101 | 20ul |  |
| DTT (dithiothreitol) solution 1M | #1117316 |  | -20°C |

## Procedure

Extraction number: _______________________________		Date:__________________________________________

Personnel: ______________________________________		Sample Information: Number of samples (=n): ________

| Sample ID | Species | Collection time | Collection Date | Collection location | DNA extraction date | Sample volume | DNA extract ID | DNA conc. (ng/ul) | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Blank | x | x | x | x |  |  |  |  |  |

Comments:

### Before starting

- Make sure that the appropriate amounts (as indicated on bottle) of ethanol were added to the AW1 and AW2 buffer concentrates.

- Prepare an aqueous 1M DTT solution. Store aliquots at -20°C. Thaw immediately before use.

- If Buffer AL or ATL have formed a precipitate, warm to 56°C to dissolve

- Preheat the heating block or thermomixer to 56°C

### Lysis

- Pipet into labelled 1.5ml tube:

300ul Buffer ATL,

20ul Proteinase K,

20ul DTT

Pulse-vortex for 10sec

- Cut off a 1cm piece from the base of the hair and transfer it into the tube.

Pulse-vortex for 10s

- Incubate 1h to overnight at 56°C shaking 900rpm.

Sample should be completely lysed, and duration might depend on the sample. Vortex in between to improve lysis

Use incubation time to prepare Spin Columns and label

- Vortex for 10s, spin down

- Add 300ul Buffer AL to the sample

Vortex

- Incubate at 70°C and with 900rpm for 10min

Spin down

- Add 150ul 99%EtOH

Mix thoroughly by vortexing to yield a homogeneous solution

Spin down

*Note: A white precipitate may form on addition of Buffer AL and ethanol. This precipitate does not interfere with the procedure. Some sample types may form a gelatinous lysate after addition of Buffer AL and ethanol. In this case, vigorously shaking or vortexing the preparation is recommended.

### Binding and Washing

- Pipet the ca. 800ul mixture into spin column

Centrifuge 8000rpm for 1min

Discard collection tube with flow-through

*Note: Centrifuge again if not all lysate has passed through the membrane. Flow-through contains Buffer AL or Buffer AW1 and is therefore not compatible with bleach.

- Place spin column into new collection tube

Add 500ul Buffer AW1

Centrifuge 8000rpm for 1min

Discard collection tube with flow-through

- Place spin column into new collection tube

Add 700ul Buffer AW2

Centrifuge 8000rpm for 1min

Discard collection tube with flow-through

- Place spin column into new collection tube

Add 700 ul EtOH (99%)

Centrifuge 8,000rpm for 1min to dry membrane

Discard collection tube with flow-through

- Place spin column into new collection tube

Centrifuge 14,000rpm for 3min to dry membrane

Discard collection tube with flow-through

*Note: it is important to dry the membrane of the spin column, since residual ethanol may interfere with subsequent reactions.

### Elution

- Place spin column into clean, labelled 1.5ml tube

Open lid, incubate 10min at RT or 3min at 56°C

- Pipet 55ul Buffer ATE onto membrane

Incubate 20min at RT

Centrifuge 14000rpm for 1min

- Again pipet 55ul Buffer ATE onto membrane

Incubate 20min at RT

Centrifuge 14000rpm for 1min

- Store DNA eluate at 2-8°C for direct further use, at -20°C for mid-term storage, at -80°C for long-term storage.

*Note: Prepare 2 aliquots of 50ul each to prevent freeze-thaw cycles in downstream use.

- Make sure to have all tubes well labelled (incl “DNA”, sample ID, DNA extract ID; date, storage buffer) and included in databases.

## Continue with:

### DNA quantification and quality

- Qubit dsDNA BR assay

    - SOP #IV.01

  - TapeStation

    - SOP XXX

*Note: Expected DNA amount ca. 5ug

### DNA library preparation and sequencing

- Send DNA to sequencing facility

    - SOP XXXX